SPECIFICATION

DESCRIPTION

CAT6 4Pair 23AWG Solid CCA	Technical Data

Outdoor Jacket UV Rated Waterproof	Rated Temperature	70 °C

Rated Voltage	30V

| FEATURES | Reference Standard | EIA/TIA 568 & ISO/IEC 11 |
| --- | --- | --- |
| - High-Performance Data Cable | Conductor | Solid CCA |
| - CAT6 Unshielded Twisted Pairs | Material | Copper Clad Aluminum |
| - Easily Identified Color-Striped Pairs | AWG | 23AWG |
| - 23AWG Solid CCA | Conductor Diameter | 4 × 2 × 1/0.56 ± 0.01mm |
| - Outdoor UV Rated Jakcet Waterproof |  |  |
|  | Insulation | HDPE |

801

APPLICATION

High Speed Horizontal Cabling

100 Mbps TPDDI , 1000 Base T , 622 Mbps ATM

AVG. Thickness	0.21mm

Insulation Diameter	1.01 ± 0.01mm

High twisted & Ripcord	Yes

IEEE802.3/IEEE802.5 , 10 Base-T , 100 Base-T4	Separator	PP

| Jacket |  | LDPE (UV) |
| --- | --- | --- |
| AVG. Thickness |  | 0.61 ± 0.01mm |
| Outer Diameter |  | 6.2 ± 0.1mm |
| Color |  | Black |
| Marking |  | Available |
| Color of Pairs Pair 1 | Blue | White-Blue |
| Pair 2 | Orange | White-Orange |
| Pair 3 | Green | White-Green |
| Pair 4 | Brown | White-Brown |

1000 Base-X , 100 Base-VG , Gigabit Ethernet

| MODEL |  |  |
| --- | --- | --- |
| No | Length | Color |
| 140201 | 1000ft(305m) | Black |

PACKING

PERFORMANCE

Wooden drum	Mechanical Properties

Outer Brown Carton	PVC Tensile Strength(Mpa)	≥ 13.8

Bending Radius	≥ 4xOD(without load) Temperature range	during operation: -40°C ~ 75°C

Electrical Properties

Max. Con. DC resistance	75 ohms/km at 21 ºC

1.0 - 100 MHz Impendance	100 ± 15 (Ohms)

100 - 250 MHz Impendance	100 ± 15 (Ohms)

250 - 550 MHz Impendance	100 ± 35 (Ohms)

1.0 - 250 MHz Delay Skew	≤ 45 Pair-to-ground Cap. Unbalance ≤ 330Resistance Unbalance	≤ 5%