DESCRIPTION

CAT6 4Pair UTP 23AWG Solid Bare Copper	Technical Data

SPECIFICATION

Outdoor Jacket UV Rated Waterproof	Rated Temperature	70 °C

Rated Voltage	30V

FEATURES

Reference Standard	EIA/TIA 568 & ISO/IEC 11801

- High-Performance Data Cable	Conductor	Solid Bare Copper

- CAT6 Shielded Twisted Pairs	Material	Bare Copper (100% Copper)

- Mylar and drain wire	AWG	23AWG

- 23AWG Solid Solid Bare Copper	Conductor Diameter	4 × 2 × 1/0.57 ± 0.01mm

- Outdoor UV Rated Jakcet Waterproof

APPLICATION

High Speed Horizontal Cabling

100 Mbps TPDDI , 1000 Base T , 622 Mbps ATM

Insulation	HDPE

AVG. Thickness	0.21mm

Insulation Diameter	1.01 ± 0.01mm

High twisted & Ripcord	Yes

IEEE802.3/IEEE802.5 , 10 Base-T , 100 Base-T4	Separator	PP

1000 Base-X , 100 Base-VG , Gigabit Ethernet	Jacket	LDPE (UV)

MODEL

AVG. Thickness	0.68 ± 0.1mm

No	Length	Color

140401	305M	Black

Outer Diameter	7.8 ± 0.1mm

Color	Black

140402	500M	Black	Marking	Available

| Shielded | Color of Pairs Pair 1 Blue |  | White-Blue |
| --- | --- | --- | --- |
| Drain Wire | Pair 2 | Orange | White-Orange |
| PE Tape ( Mylar ) | Pair 3 Pair 4 | Green Brown | White-Green White-Brown |

HSD CABL	WWW.HSDCABLE.COM

CAT6 OUTDOOR UNSHIELDED UV RATED

Email : sales@hsdcable.com Tel : +86 135288hsdcable.en.alibaba.com

@ Copyright 2013-2020 Hsdcable Ltd . All rights reserved

PACKING

PERFORMANCE

Wooden drum	Mechanical Properties

Outer Brown Carton	PVC Tensile Strength(Mpa)	≥ 13.8

Bending Radius	≥ 4xOD(without load) Temperature range	during operation: -40°C ~ 75°C

Electrical Properties

Max. Con. DC resistance	75 ohms/km at 21 ºC

1.0 - 100 MHz Impendance	100 ± 15 (Ohms)

100 - 250 MHz Impendance	100 ± 15 (Ohms)

250 - 550 MHz Impendance	100 ± 35 (Ohms)

1.0 - 250 MHz Delay Skew	≤ 45 Pair-to-ground Cap. Unbalance ≤ 330 Resistance Unbalance	≤ 5%

HSD CABLE	WWW.HSDCABLE.COM